PUBLIC WATER AND WASTEWATER IMPROVEMENTS GENERAL NOTES

1. PRIOR TO ANY CONSTRUCTION, THE CONTRACTOR SHALL FAMILIARIZE HIMSELF WITH THE CONSTRUCTION DOCUMENTS, THE PLANS INCLUDING ALL NOTES, THE CITY OF WEATHERFORD SPECIFICATIONS, AND ANY OTHER APPLICABLE STANDARDS OR SPECIFICATIONS RELEVANT TO THE PROPER COMPLETION OF THE WORK SPECIFIED.  FAILURE ON THE PART OF THE CONTRACTOR TO FAMILIARIZE HIMSELF WITH ALL THE STANDARDS OR SPECIFICATIONS PERTAINING TO THIS WORK SHALL IN NO WAY RELIEVE THE CONTACTOR OF RESPONSIBILITY FOR PERFORMING THE WORK IN ACCORDANCE WITH ALL SUCH APPLICABLE STANDARDS AND SPECIFICATIONS.

2. THE CONTRACTOR SHALL HAVE IN HIS POSSESSION, PRIOR TO CONSTRUCTION, ALL NECESSARY PERMITS, LICENSES, ETC.  THE CONTRACTOR SHALL HAVE AT LEAST ONE SET OF APPROVED ENGINEERING PLANS AND SPECIFICATIONS ON SITE AT ALL TIMES.

3. THE PROPOSED LINES AT TIMES WILL BE LAID IN CLOSE PROXIMITY TO OTHER EXISTING UTILITIES AND STRUCTURES BOTH ABOVE AND BELOW GROUND.  THE CONTRACTOR SHALL MAKE NECESSARY PROVISIONS FOR THE SUPPORT  AND PROTECTION OF ALL UTILITY POLES, FENCES, TREES, SHRUBS, GAS MAINS, TELEPHONE CABLES, POWER LINES, DRAINAGE PIPES, UTILITY SERVICES, AND ALL OTHER UTILITIES AND STRUCTURES BOTH ABOVE AND BELOW GROUND DURING CONSTRUCTION.  IT IS THE CONTRACTOR'S RESPONSIBILITY TO NOTIFY ALL UTILITY OWNERS PRIOR TO ANY CONSTRUCTION IN THE AREA AND VERIFY THE ACTUAL LOCATION OF ALL BURIED UTILITIES THAT MAY OR MAY NOT BE SHOWN ON THE PLANS. THE CONTRACTOR SHALL PRESERVE AND PROTECT ALL UNDERGROUND AND OVERHEAD FACILITIES AND BE RESPONSIBLE FOR ANY DAMAGE CAUSED TO THEM.

4. THE CONTRACTOR SHALL VERIFY THE ELEVATION, CONFIGURATION, AND ANGULATIONS OF EXISTING LINES PRIOR TO CONSTRUCTION OF TIE-IN MATERIALS.

5. ALL MATERIALS AND CONSTRUCTION SHALL CONFORM TO CITY OF WEATHERFORD SPECIFICATIONS, THE PROJECT DETAILS AND SPECIFICATIONS.  IN THE EVENT THAT AN ITEM IS NOT COVERED IN THE CONTRACT DOCUMENTS, THE ENGINEER'S DECISION WILL APPLY.

6. TRENCH SAFETY DESIGN SHALL BE THE RESPONSIBILITY OF THE CONTRACTOR.

7. THE CONTRACTOR SHALL BE RESPONSIBLE FOR PREPARATION, IMPLEMENTATION AND MAINTENANCE OF A TRAFFIC CONTROL PLAN.  THE TYPES AND LOCATIONS OF THE TEMPORARY BARRICADES AND SIGNS USED DURING CONSTRUCTION SHALL CONFORM TO THE LATEST VERSION OF THE TEXAS MANUAL ON UNIFORM TRAFFIC CONTROL DEVICES.

8. DRIVEWAYS SHALL REMAIN OPEN AT ALL TIMES.  CONTRACTOR SHALL COORDINATE WITH PROPERTY OWNERS REGARDING ANY TEMPORARY DRIVEWAY CLOSINGS.

9. NO EXCAVATED MATERIALS, BACKFILL MATERIALS, EQUIPMENT, OR SUPPLIES SHALL BE STORED WITHIN FLOODWAYS, DRAINAGE EASEMENTS, OR TXDOT RIGHT OF WAY.

10. ALL FENCES, DRIVEWAYS, MAILBOXES, LANDSCAPING IRRIGATION SYSTEMS, CULVERT PIPES, DRAINAGE DITCHES,  PRIVATE YARDS, ROADWAYS, PARKING LOTS OR OTHER IMPROVEMENTS NOT SCHEDULED FOR REPLACEMENT DURING CONSTRUCTION WHICH ARE DAMAGED OR MOVED BY THE CONTRACTOR SHALL BE RESTORED TO ORIGINAL CONDITION OR BETTER CONDITION BY THE CONTRACTOR.

11. THE ALIGNMENTS HAVE BEEN DESIGNED TO MINIMIZE THE REMOVAL OF TREES. THE CONTRACTOR SHALL NOTIFY THE OWNER'S PROJECT REPRESENTATIVE PRIOR TO REMOVING ANY TREES. NO TREES SHALL BE REMOVED WITHOUT OWNER APPROVAL, UNLESS SHOWN ON PLANS.

12. THE CONTRACTOR SHALL BE RESPONSIBLE FOR MAINTAINING GENERAL SAFETY AT AND ADJACENT TO THE PROJECT AREA, INCLUDING THE PERSONAL SAFETY OF THE CONSTRUCTION CREW AND GENERAL PUBLIC AND THE SAFETY OF PUBLIC AND PRIVATE PROPERTY.

13. THE CONTRACTOR IS RESPONSIBLE FOR KEEPING STREETS AND SIDEWALKS ADJACENT TO THE PROJECTS FREE OF MUD AND DEBRIS FROM THE CONSTRUCTION. THE CONTRACTOR SHALL BE RESPONSIBLE FOR MAINTAINING THE CLEANLINESS OF STREETS AND SIDEWALKS WITHIN AND ADJACENT TO THE PROJECT AREA, ENSURING THEY REMAIN FREE FROM MUD AND DEBRIS ATTRIBUTABLE TO CONSTRUCTION ACTIVITIES. THE CONTRACTOR SHALL SWEEP AND REMOVE DUST, DIRT, ROAD BASE, ETC. AS FREQUENTLY AS NECESSARY TO MAINTAIN A CLEAN WORK SITE AS DETERMINED BY THE CITY INSPECTOR.

14. THE CONTRACTOR SHALL SAWCUT EXISTING CURB AND GUTTER, PAVEMENT, DRIVEWAYS, ALLEYS AND SIDEWALKS AT AREAS WHERE PAVEMENT OR CONCRETE IS TO BE REMOVED.  THE CONTRACTOR SHALL PROVIDE COMPACTED SELECT BACKFILL MATERIAL AS NEEDED TO REPAIR DAMAGED STREETS, DRIVEWAYS, WALKS, PARKING LOTS, ETC.

15. CONCRETE BLOCKING SHALL BE INSTALLED ON ALL WATER LINE BENDS, FIRE HYDRANTS, FITTINGS AND VALVES.  ALL HORIZONTAL BLOCKING, CRADLE BLOCKING AND VERTICAL TIE-DOWN BLOCKING SHALL BE IN ACCORDANCE WITH THE PLAN DETAILS.  BLOCKING SHALL BE SUBSIDIARY TO WATER LINE CONSTRUCTION AND NO EXTRA COMPENSATION WILL BE ALLOWED.

16. ELEVATION ADJUSTMENTS OF WATER LINE AT CONNECTIONS MAY BE MADE WITH BENDS, OFFSETS, OR JOINT DEFLECTIONS.  ALL NONSTANDARD BENDS SHALL BE MADE USING THE CLOSEST STANDARD MJ FITTINGS WITH THE REQUIRED JOINT DEFLECTIONS.  DEFLECTIONS SHALL NOT EXCEED THE MANUFACTURER'S RECOMMENDED DEFLECTION PER JOINT.

17. UNLESS OTHERWISE SHOWN, ALL WATER LINES SHALL BE PRESSURE TESTED AND DISINFECTED IN ACCORDANCE WITH NCTCOG SPECIFICATIONS.  WATER LINES SHALL HAVE A MINIMUM OF 42" OF COVER UNLESS OTHERWISE NOTED.

18. ALL WATER SERVICES SHALL BE 1" TYPE K COPPER PIPE UNLESS OTHERWISE NOTED.

19. CURB STOPS SHALL BE TESTED FOR LEAKAGE AND FULL FLOW WHEN THE SYSTEM IS PRESSURE TESTED.

20. ALL FIRE HYDRANTS ARE TO BE EQUIPPED WITH A SIX (6) INCH GATE VALVE AND BOX. ALL VALVES AND FIRE HYDRANTS ARE TO BE PER CITY STANDARD SPECIFICATIONS.  HYDRANTS SHALL BE PAINTED AS FOLLOWS:  2 COATS Sherwin Williams International Red – F75RC7 OVER SHOP PRIME COAT FOR BONNET AND END CAPS.

21. IN ORDER TO MINIMIZE SERVICE DISRUPTIONS, THE CONTRACTOR SHALL COORDINATE ALL CONNECTIONS TO EXISTING WATER AND SEWER LINES WITH THE CITY'S INSPECTOR.  NO CONNECTIONS TO EXISTING WATER AND/OR WASTEWATER FACILITIES WILL BE ALLOWED WITHOUT THE APPROVAL OF THE CITY INSPECTOR.

22. THE CONTRACTOR SHALL PROVIDE CLOSURE PIECES FOR PIPELINES AS REQUIRED TO CONSTRUCT THE PROJECT, INCLUDING THOSE REQUIRED FOR SPECIAL CONSTRUCTION PROCEDURES TO COORDINATE CHANGES TO THE SEQUENCE OF CONSTRUCTION.

23. THE CONTRACTOR SHALL BE REQUIRED TO INSTALL TEMPORARY PLUGS FOR HYDROSTATIC TESTING AS NECESSARY.

24. THE CONTRACTOR SHALL OBTAIN A PERMIT FROM THE CITY OF WEATHERFORD DEPARTMENT OF TRANSPORTATION AND PUBLIC WORKS PRIOR TO ANY CONSTRUCTION WITHIN THE CITY RIGHT OF WAY.

25. THE CONTRACTOR SHALL BACKFILL ALL OPEN TRENCHES AS SOON AS POSSIBLE AND IN NO CASE SHALL UNPROTECTED TRENCHES REMAIN OPEN DURING NON-WORKING HOURS.

26. ALL TRENCH BACKFILL IN UNPAVED AREAS SHALL BE PLACED TO EXISTING GRADE PLUS SIX INCHES TO ALLOW FOR SETTLEMENT AND PREVENT PONDING OF STORM WATER RUNOFF DUE TO CONSTRUCTION ACTIVITIES UNLESS PROPOSED FINISHED GRADE IS SHOWN.

27. THE CONTRACTOR SHALL BE RESPONSIBLE FOR PROVIDING ADEQUATE MEASURES FOR PREVENTING STORM WATER RUNOFF FROM ENTERING THE TRENCH DURING CONSTRUCTION.

28. ALL EXCAVATIONS, TRENCHING AND SHORING OPERATIONS SHALL COMPLY WITH THE REQUIREMENTS OF THE U.S. DEPARTMENT OF LABOR, OSHA "CONSTRUCTION SAFETY AND HEALTH REGULATIONS," VOLUME 29, SUBPART P, PAGES 128-137, AND ANY AMENDMENTS THERETO.

29. THE CONTRACTOR SHALL TAKE ADEQUATE MEASURES TO PREVENT EROSION.  IN THE EVENT THAT SIGNIFICANT EROSION OCCURS AS A RESULT OF CONSTRUCTION, THE CONTRACTOR SHALL RESTORE THE ERODED AREA TO ORIGINAL CONDITION.

30. THE CONTRACTOR SHALL NOT OPERATE ANY EXISTING WATER VALVES UNLESS THE CITY INSPECTOR IS PRESENT.

31. THE CONTRACTOR SHALL VIDEO ALL POTENTIALLY IMPACTED PROPERTY AREAS PRIOR TO WORK.  VIDEOS SHALL INCLUDE DATE, NOTATION AND AUDIO IDENTIFICATION OF PROPERTY ADDRESS AND PROJECT NAME.  THE CONTRACTOR SHALL PROVIDE ONE COPY OF THE VIDEO FILE OR DVD TO THE CITY'S INSPECTOR PRIOR TO CONSTRUCTION.

32. INITIAL TESTING OF EMBEDMENT AND BACKFILL MATERIAL SHALL BE BY THE CITY AT ITS OWN EXPENSE.  ANY RETESTS DUE TO FAILURE SHALL BE AT THE CONTRACTOR’S EXPENSE.

33. THE CONTRACTOR IS RESPONSIBLE FOR ALL CONSTRUCTION STAKING.

34. THE CONTRACTOR SHALL BE RESPONSIBLE FOR DISPOSING OF ALL EXCESS EXCAVATION AND SPOIL IN ACCORDANCE WITH CITY, STATE AND FEDERAL GUIDELINES.

35. THE CONTRACTOR'S PERSONNEL SHALL WEAR IDENTIFYING CLOTHING AND HATS AT ALL TIMES.

36. THE CONTRACTOR SHALL:

a) MAINTAIN A MINIMUM OF ONE FOOT (1') CLEARANCE FROM ALL UTILITIES, UNLESS OTHERWISE NOTED ON THE PLANS;

b) MAINTAIN NEAT AND ACCURATE RECORD PLANS OF CONSTRUCTION, INDICATING LAYOUT OF WORK AND ITS RELATION TO UTILITIES IN THE FIELD;

c) MAINTAIN A CLEAN WORK SITE AND DAILY REMOVE ALL TRASH AND DEBRIS;

d) ENSURE THAT ALL LANDSCAPE ELEMENTS, INCLUDING, BUT NOT LIMITED TO, ROCKS, PAVERS AND PLANTERS ARE RESTORED TO THEIR EXISTING CONDITION AT THE COMPLETION OF THE WORK.  ANY DAMAGED ELEMENTS SHALL BE REPLACED IN KIND BY THE CONTRACTOR AT HIS EXPENSE;

e) RESTORE GRASSED AREAS TO THEIR ORIGINAL GRADE AND VEGETATIVE CONDITION. ALL DISTURBED AREAS SHALL BE HYDRO-MULCH SEEDED OR SOD AS STATED ON THE PLANS, 20' ON EACH SIDE OF THE PIPELINE ALIGNMENT.  THIS SHALL BE SUBSIDIARY TO THE COST OF THE PROJECT.

f) GIVE ANY AFFECTED RESIDENT AT LEAST 48 HOURS NOTICE PRIOR TO ANY TEMPORARY BLOCKING, REMOVAL OR REPLACEMENT OF DRIVEWAY APPROACHES;

g) REPAIR ANY DAMAGE TO PAVEMENT THAT IS OUTSIDE THE ESTABLISHED WORKING AREA AND WAS CAUSED BY THE CONTRACTOR'S OPERATIONS.

h) SHALL RESTORE ALL DRIVEWAYS AND SURFACES ON PUBLIC AND PRIVATE PROPERTY TO MAINTAIN PROPER ACCESS TO THEIR PROPERTY OUTSIDE OF STANDARD WORKING HOURS.

37. SPOILS AND OTHER MATERIALS TO BE REMOVED FROM THE PROJECT SITE THAT ARE TEMPORARILY STORED IN THE RIGHT OF WAY SHALL BE REMOVED WEEKLY AT A MINIMUM.

38. CONSTRUCTION VEHICLES, EQUIPMENT, AND MATERIALS STORED IN THE RIGHT OF WAY MUST BE LOCATED ON THE BLOCK WHERE WORK IS ACTIVELY OCCURING OR AS NEAR TO IT AS POSSIBLE. APPROPRIATE TRAFFIC CONTROL AND/OR EROSION CONTROL SHALL BE IN PLACE FOR ANY ITEM(S) STORED IN THE RIGHT OF WAY.

39. SAW CUTS AND PAVEMENT REPAIRS FOR UTILITY TRENCHES SHALL BE MADE AT UNIFORM WIDTHS UNLESS OTHERWISE APPROVED BY THE CITY INSPECTOR.